������������������������������������Ŀ

�    PCMCIA  NE2000  LAN adapter     �

�             Installation           �

��������������������������������������

�������Ŀ

�Section�

���������

1.   Summary (Quick Installation)

2.   Solutions for Intel 82365SL PCIC controller

3.   Solutions for Databook's TCIC-2/N controller

4.   Solution for Toshiba T3300SL ICCNT controller

5.   Installing SystemSoft's CardSoft

5.1  Installing SystemSoft's CardSoft for Intel 82365SL chipset

5.2  Installing SystemSoft's CardSoft for Databook chipset

5.3  Parameter explanations

6.   Installing Databook's CardTalk

7.   Installing Phoenix's PCM (Plus)

7.1  Installing Phoenix's PCM for Intel 82365SL chipset

7.2  Installing with Phoenix's PCM for Databook chipset

7.3  Installing Phoenix's PCM Plus for Intel 82365SL chipset

7.4  Installing Phoenix's PCM Plus for Databook chipset

7.5  Parameter explanations

A.   Syntax of ECARD.EXE

B.   Solutions for other PCMCIA controllers

���������Ŀ

�1 Summary�

�����������

The PCMCIA NE2000 Compatible is an Ethernet card that follows the PCMCIA

(Personal Computer Memory Card International Association) Release 2 typeII

standard. Following are the characteristics of PCMCIA:

-  Support for computers equipped with PCMCIA Release 2 Type II

I/O slot.

-  Support for Socket Services v1.01, v2.0, v2.01, and v2.10.

-  Support for Card Services v2.0, v2.01, and v2.10.

-  Support for Intel 82365SL PCIC, Databook TCIC-2/N, Toshiba

T3300 ICCNT or Cirrus PCMCIA chips.

-  Support for Databook' CardTalk, Phoenix's PCM (Plus) or

SystemSoft's CardSoft PCMCIA solutions.

-  Supporting I/O address range 300h - 360h, default 300h.

-  Supporting memory address range C000h ,C800h,D000h,D800h.

-  Supporting IRQ 3,4,5,9,10,12,15 default 3.

-  Providing software support for:

DEC Pathworks for DOS

IBM LAN support program v2.1x or above

IBM LAN Server/Requester

FTP PC/TCP

Microsoft LAN Manager

Microsoft Windows for Workgroups v3.1x

Novell NetWare

Novell NetWare Lite

SUN PC NFS

Support IPX or ODI NE2000 Compatible interface software

Support NDIS interface software

Support Packet Driver interface software

Wollongong Pathway

We provide PCMCIA enable programs for the LAN card as following:

��������������������������������������������Ŀ

�1.1 PCMCIA controller's (hardware) enablers:�

����������������������������������������������

Following programs can enable the PCMCIA hardware controller...

Example:

\ECARD\ECARD.EXE       :  Enables the PCMCIA Socket Services

\ECARD\ECARDHP.EXE     :  Enables the PCMCIA Socket Services for HP

1.2 PCMCIA Socket Service (software) enabler:

\ECARD\ECARD.EXE       :  Enables the PCMCIA Socket Services

interface (provided by notebook vendor)

This program enables the following software provided by notebook vendor :

(a) Socket Service (b) Databook's Cardtalk version 2.20.4 and above.

1.3 Client Driver:

\ECARD\ECARDCS.EXE    : ECARDCS.EXE is an integration version of open

adapter utility. ecard supports Intel 82365SL

, Toshiba T3300SL's ICCNT,and Socket Services

v1.01, v2.0, v2.01, V2.10,and Card   Services

v2.0, v2.01, v2.10.

In general,your notebook vendors will provide the PCMCIA system includes

the PCMCIA's Socket Service and Card Service solution.

e.g. Phoenix's PCM (Plus),  SystemSoft's  CardSoft  and so on. So,you just

need to load one of the AC drivers which listed in 1.3.

1.4 Quick installation

If you don't know what is your PCMCIA system, you can refer to following

quick installation:

1. Running the ECARD.EXE file, refer to ecard.hlp or type ecard /?.

2. Running the ne2000 compatible driver for Network Operation System.

����������������������������������������������Ŀ

�2. Solutions for Intel 82365SL PCIC controller�

������������������������������������������������

If your notebook is equipped with an Intel 82365SL PCIC controller, there

are four methods to use the PCMCIA:

2.1 Method 1:

If your notebook vendor does't provide either

Socket Service nor Card Service software:

Use the ECARD.EXE to enable Intel 82365SL Card Interface Controller(PCIC)

chipset.  ECard.exe located at the  \ECARD  subdirectory  of this diskette.

Following is an example of how to use ECARD.EXE with the ODI driver:

ECARD /io=300 /mem=d0 /irq=5

lsl

ne2000 (NOVELL NE2000 compatible driver)

ipxodi

netx

*NOTE:mem=d0(d000h)

2.2 Method 2:

If your notebook vendor provides the Socket Services , Card Services  and

other PCMCIA-supported software:

Some examples of this PCMCIA-supported software and vendors that supplied

are : SystemSoft's CardSoft , Databook's CardTalk and Phoenix's PCM (Plus).

Please refer to sections 5, 6, and 7 described later.

2.3 Method 3:

If your notebook vendor provides version v1.01, v2.0, v2.01, and v2.10 of

the Socket Services software:

Execute the  ECARD.EXE Socket Services enabler program  to initialize the

PCMCIA socket in your notebook. ECard.exe located at \ECARD subdirectory.

2.4 Method 4:

If your notebook vendor  provides  version v2.0, v2.01, and v2.10  of the

Card Services software:

Execute  the ECARD.EXE Socket Services enabler program to initialize  the

PCMCIA socket in your notebook. ECard.exe located at \ECARD subdirectory.

������������������������������������������������Ŀ

�3. Solutions for Databook's TCIC-2/N controller �

��������������������������������������������������

There are two methods for using PCMCIA with Databook TCIC-2/N controller:

3.1 Method 1:

If your  notebook vendor  provides  PCMCIA-supported  software  such  as

Card Service, Socket Service and Client Driver.  Typical  examples of this

software are SystemSoft's CardSoft and Phoenix's PCM (Plus).

Please refer to sections 5 and 7 or the manual of your notebook.

3.2 Method 2:

If your notebook vendor provides Databook's Cardtalk software:

Please execute the \ECARD\ecard.exe program to enable the Socket Services

through the CardTalk software.

������������������������������������������������Ŀ

�4. Solution for Toshiba T3300SL ICCNT controller�

��������������������������������������������������

If your notebook is equipped with a Toshiba T3300SL ICCNT controller...

Your notebook vendor  does not provide  any  PCMCIA-supported  software.

Our  ecard.exe  can  enable  the  ICCNT controller  directly  without any

PCMCIA-supported software. Refer the following examples.

ecard

lsl

ne2000  (NOVELL NE2000 Compatible)

ipxodi

netx

�����������������������������������Ŀ

�5. Installing SystemSoft's CardSoft�

�������������������������������������

SystemSoft's  CardSoft provides  Socket Service ,  Card Service and Card

Installation  Client drivers.  CardSoft supports  Intel  82365SL PCIC  and

Databook TCIC-2/N  and  other PCMCIA controllers.  Each  of  these  PCMCIA

controllers requires different Socket Service driver program.

5.1  Installing SystemSoft's CardSoft for Intel 82365SL chip

Required CardSoft's programs for Intel 82365SL PCIC chip :

SS365SL.EXE  : Socket Service driver

CS.EXE       : Card Service driver

CIC.EXE      : Card Installation Client driver

Example: installing SystemSoft's CardSoft for ODI driver:

The CONFIG.SYS file should include the following statements

:

DEVICE=SS365SL.EXE

DEVICE=CS.EXE

DEVICE=CIC.EXE  /LIRQ 5 /NWBAS D0 /BAS D8

:

The AUTOEXEC.BAT file should include the following statements

:

lsl

ne2000        ;NE2000 Compatible

ipxodi

netx

:

5.2  Installing SystemSoft's CardSoft for Databook chipset

Required CardSoft's programs for Databook TCIC-2/N chip :

SSDBOOK.EXE  : Socket Service driver

CS.EXE       : Card Service driver

CIC.EXE      : Card Installation Client driver

Example: installing SystemSoft's CardSoft for NDIS driver:

The CONFIG.SYS file should include the following statements

:

DEVICE=SSDBOOK.EXE

DEVICE=CS.EXE

DEVICE=CIC.EXE  /LIRQ 5 /NWBAS D0 /BAS D8

rem

rem    NDIS driver

rem

device=protman.dos [/i:<path name of the PROTOCOL.INI file>]

device=ne2000.DOS        ;real mode ne2000 compatible driver

:

The AUTOEXEC.BAT file should include the following statements

:

netbind

{your network operation system}

:

5.3  Parameters explanations

SS365SL.EXE :

/PORT hhh  Specifies the  base port address  of the first of two PCMCIA

Adapter boards. SS386SL assumes the each Adapter responds to

two consecutive I/O ports for its address/data ports and that

all of four ports are consecutive if two Adapters are present.

The default port address are 3E0h/3E1h for the first Adapter

and 3E2h/3E3h for second Adapter (/PORT 3E0).

/SKT n     Specifies the number of supported sockets (1-4). The default

is 2 (/SKT 2).

CS.EXE      :

/IRQ h     Specifies the IRQ number to be used for the card-status-change

interrupt. The default value is 0Fh (/IRQ F). The allowable

range is 0h through 0Fh.

CIC.EXE     :

/BAS hh    Specifies the  base address  in  system memory  of  the memory

window used to access Attribute and Common Memory space during

card recognition and initialization. The value  must be states

in 4K units.The default value is D0h (/BAS D0) which specifies

a window string at D0000:0h.The base address specified must be

in the range C0 through EFh.

/SIZ hh    Specified the size of the  system  memory window. The size is

stated in 4K units (where 1=4K,2=8K,3=12K, etc.). The default

is 4h (/SIZ 4) which signifies a 16K window.

/CSBAS hh  Specifies the base segment address of a  window area given to

Card  Services with an  AdjustResoureInfo function  call. The

value must be in 4K units.The default value is D4h (/CSBAS D4)

, which specifies  a window  starting  at D400:000h. The base

address specified must be in the range C0h to EFh.

/CSSIZ hh  Specifies the size of a window area to  Card Services with an

AdjustResourceInfo  function  call. The size is stated by  4K

units (where 1=4K,2=8K,3=12K,etc.) with 64K being the maximum

size. The default value is 4h ( /CSBAS 4 ), which signifies a

16K window.If the specified value is 0,the AdjustResourceInfo

function call is not made.

/NWBAS hh  Specifies  the base segment address of a attribute or  common

memory tobe opened. The value must be in 4K units.The default

value is D8h ( /NWBAS D8 ), which specifies a window starting

at D800:0h. The  base address specified must be  in the range

C0h through EFh. This switch  applies to network  cards  that

require an attribute or common memory window  to be left open

after configuration.

/NWSIZ hh  Specifies the size of memory window to be left open. The size

is stated in 4K units (where 1=4K, 2=8K, 3=12K,etc.) with 64K

being the maximum size. The default value is 4h ( /CSBAS 4 ),

which  signifies  a 16K window. The applies to  network cards

that require  an attribute or common memory window to be left

open after configuration.

/LIRQ hh   Specifies  the IRQ that is assigned to LAN cards. The default

is 5h (/LIRQ 5). The allowable range is 0h through 0Fh.

[Notes]

* Make sure that the memory windows (/BAS, /CSBAS, /NWBAS) did't overlap.

The default value of these switches prevent memory window address conflict.

Use cautiously if changing the default values. By the other hand,if you use

some memory management software,please avoid the memory conflict with above

memory windows. For example: if you use the Microsoft's EMM386.EXE,  please

attach with "X=D000-DBff" parameter (for the default values of /BAS, /CSBAS,

/NWBAS).

���������������������������������Ŀ

�6. Installing Databook's CardTalk�

�����������������������������������

Here  provides a  Socket Services Enabler  program called  ECard.exe for

running with  Databook's CardTalk.  Databook's Socket Service and CardTalk

must first be installed in your notebook.

To run PCMCIA with the Databook's CardTalk on Databook TCIC-2/N controller

1. Check the Socket Service driver that was supplied by  either the ROM

BIOS extension or a loadable file (normally named *PV.SYS).

* The PC or Notebook vendor have to provide the Socket Service driver

(either ROM BIOS extension or loadable file) to initialize PCMCIA.

2. Find out the CARDTALK.SYS file to provide  an interface between  the

Socket Service driver and Socket Service enabler ECard.exe. It  will

enable the Socket Service through CARDTALK.SYS.

3. Modify your CONFIG.SYS and AUTOEXEC.BAT files like following examples

CONFIG.SYS file:

:

DEVICE=*PV.SYS        (or loaded by ROM BIOS extension)

DEVICE=CARDTALK.SYS

DEVICE=ecard.exe  /IRQ=5 /MEM=D0 /IO=300 /SKT=1

:

AUTOEXEC.BAT file:

:

lsl

ne2000        ;Using ne2000 compatible

ipxodi

netx

:

[Notes]

*  If you want to change the IRQ , I/O or MEMORY for NE2000 driver , you

only need to adjust the parameters of the ecard file for PCMCIA.

*  Databook's TCIC-2/N chip doesn't support IRQ 9, so if you install for

this controller, do not use IRQ 9.

������������������������������������Ŀ

�7.   Installing Phoenix's PCM (Plus)�

��������������������������������������

Phoenix provides the following PCMCIA solutions:

PCM (Phoenix PC Card Manager), PCM Plus, and others.

PCM:

The PCM  solution  complies  with  PCMCIA Socket Service  and  ExCA 1.10

specifications. It provides the ExCA Socket Service , Card Configuration

Utility and Client drivers (for IBM, etc.).

PCM Plus:

The PCM Plus solution complies with PCMCIA Reversion 2.0 Socket Services

& Card Services specifications (also incorporate the ExCA specification).

It provides  the PCMCIA Revision 2.0 Socket Services , Card Services and

Client driver.

PCM and PCM Plus support Intel 82365SL PCIC and Databook TCIC-2/N, etc...

7.1  Installing Phoenix's PCM for Intel 82365SL chipset

Required Phoenix PCM software for Intel 82365SL chipset :

PCMSS.EXE       :ExCA Socket Services driver for Intel PCIC

PCMCCU.EXE      :PCMCIA Card Configuration Utility for Intel PCIC

PCMLINK.COM     :PCMCIA card Client driver for PCMCIA

* This file is provided by Phoenix,

it only supports these fixed parameters:

IO:320h, IRQ: 5, MEM: D6000h.

* Note:

For PCMCCU ver. 1.3.1 and later, the client driver is already included,

so, you don't need to run the PCMLINK.COM file.

But for PCMCCU ver. 1.3, you need to run the PCMLINK.COM program before

installing PCMCIA driver program.

Example: (for ODI driver)

The content of CONFIG.SYS should include the following statements:

:

device=PCMSS.EXE /irq=f

device=PCMCCU.EXE

:

The content of AUTOEXEC.BAT should include the following statements:

:

[ PCMLINK ]      ;needed if PCMCCU version is 1.3 or earlier

lsl

ICARDODI         ;Using I/O 320h, IRQ 5, MEMORY D600h in NET.CFG

ipxodi

netx

:

7.2  Installing for Phoenix's PCM for Databook chipset

Required Phoenix PCM software for Databook TCIC-2/N chipset :

PCMSS.EXE     :Socket Services driver for Databook TCIC-2/N

PCMCCU.EXE    :PCMCIA Card Configuration Utility for Databook TCIC-2/N

PCMLINK.COM   :PCMCIA card Client driver for Phoenix

* Note:

For PCMCCU ver. 1.3.1 and later, the client driver is already included,

so, you don't need to run the PCMLINK.COM file.

But for PCMCCU ver. 1.3,you have to run the PCMLINK.COM program before

installing PCMCIA driver program.

Example: (for IPX driver)

The content of CONFIG.SYS should contain the following statements:

:

device=PCMSS.EXE /irq=f

device=PCMCCU.EXE

:

The content of AUTOEXEC.BAT should contain the following statements:

:

[ PCMLINK ]      ;Needed if PCMCCU version is 1.3 or earlier

NE2000           ;NE2000 Compatible Driver

NETX

:

7.3  Installing Phoenix's PCM Plus for Intel 82365SL chipset

Required Phoenix PCM Plus software for Intel 82365SL PCIC chipset :

PCMSS.EXE       : Socket Services driver for Intel 82365SL PCIC

PCMCS.EXE       :PCMCIA Card Services driver

PCMSCD.EXE      :PCM Plus Super Client Driver

Example: (for ODI driver)

The content of CONFIG.SYS should contain the following statements:

:

device=PCMSS.EXE /s0=2

device=PCMCS.EXE /addr=d0

device=PCMSCD.EXE /LANIRQ=5

:

The content of AUTOEXEC.BAT should contain the following statements:

:

lsl

NE2000        ;NE2000 Compatible driver

ipxodi

netx

:

7.4  Installing Phoenix's PCM Plus for Databook chipset

Required Phoenix PCM Plus software for Databook TCIC-2/N chipset :

PCMSS.EXE       : Socket Services driver for Databook TCIC-2/N

PCMCS.EXE       : PCMCIA Card Services driver

PCMSCD.EXE      : PCM Plus Super Client Driver

Example: (for IPX driver)

The content of CONFIG.SYS should include the following statements:

:

device=PCMSS.EXE /s0=2

device=PCMCS.EXE /addr=d0

device=PCMSCD.EXE /LANIRQ=5

:

The content of AUTOEXEC.BAT should include the following statements:

:

lsl

NE2000         ;NE2000 Compatible IPX Driver

netx

:

7.5  Parameter explanations for reference

PCMSS.EXE  :

/S0=n      Specifies number of the sockets. Default vsalue is 2.

PCMCS.EXE  :

/ADDR=n    Specifies  the internal memory windows  for Card Services

to address xx00:0000. (You have to specify this parameter

when you use I/O card (e.g., PCMCIA)

PCMSCD.EXE :

/LANIRQ=d  Specifies the IRQ number to be used for LAN card.

The default value is 5. The allowable range is 0 through 15

[Notes]

*  After we tested with PCM Plus solution, we found out:

When you running  I/O card with  PCMCS.EXE , you have to specify the

parameter /ADDR=xx with PCMCS.EXE to open  an internal memory window

for Card Services,  the memory window can't conflict with the memory

window of the PCMCIA.

��������������������Ŀ

�A. Syntax of ecard  �

����������������������

**********************

Cilent Driver: ECARDCS

**********************

ECardCS.exe is an integrated Client driver of open adapter utility.

It supports Intel 82365SL, Toshiba T3300SL's ICCNT, and

Socket Service v1.01, v2.0, v2.01, v2.10,

Card Services v2.0, v2.01, v2.10.

ECardCS will auto-detect the PCMCIA system to check out which PCMCIA

interface is ready for use and  open the I/O window mapping , memory

window mappin and IRQ for PCMCIA card.

One method can be used to execute ecardcs:

Method 1

--------

From DOS prompt,

A>ecardcs

Example:

A>ecard /mem:d000 /irq:5 /io:300

A>ecardcs

Tested Compatible Model List

============================

PCM+ Card Services (v2.00, v2.01, v2.10)

PCM+ Socket Services (v2.00, v2.01, v2.10)

Systemsoft Card Services (v2.00, v2.01, v2.10)

Systemsoft Socket Services (v2.00, v2.01, v2.10)

Award on Zenith SystemsLite machine (same as Ventura)

Ventura Micro Card Services (v2.00, v2.01, v2.10) on Zenith SystemsLite

Ventura Micro Socket Services (v2.00, v2.01, v2.10)

AMI Socket Services (v1.01)

IBM Socket Services (v1.01, v2.0) on IBM ThinkPad 720

IBM Socket Services v1.01 on IBM ThinkPad 710

IBM Card Services v2.0 on IBM ThinkPad 720

Databook CardTalk Socket Services (v1.01)

Intel PCIC adapter

Toshiba ICCNT adapter.

������������������������������������������Ŀ

� B. Solutions for other PCMCIA controllers�

��������������������������������������������

(Examples with IPX, ODI, NDIS, Packet Driver with PCMCIA systems)

-----------------------------------------------------------------

When you use a PCMCIA system that not listed above , please refer  to the

following steps to find out the solution:

C.1 Support Socket Services interface

Use the ecard program we provide to enable the Socket Services interface,

then run one of the driver program for your network operating system.

C.2 Support Card Services interface

Use the ecard program we provide to enable the Card   Services interface,

then run one of the driver program for your network operating system.

C.3 Support total PCMCIA solution that includes the Socket

Services, Card Services and Client driver:

Examples:

1. IPX driver:

>ECARD.EXE

> NE2000      ;IPX driver

2. ODI driver:

>ECARD.EXE

> lsl

> NE2000      ;with NET.CFG for configuration

> ipxodi

3. NDIS driver:

device=PROTMAN.DOS [/i:<path name of the PROTOCOL.INI file>]

device=NE2000.DOS   ;with PROTOCOL.INI for configuration

> netbind

4. Packet Driver:

> ne2000 compatible